ASSIGNMENT #02 – CLASS ACTIVITY

ROS 2-Based Line-Sensing DC Motor Control System

Actuators, Sensors and Signals – Fall Semester 2026  |  School of IT & Engineering (SITE)

Instructor: Prof. Muhammad Ilyas, Ph.D. in Robotics Engineering

Project Team: Maratova Aissulu, Ailenuer Anala, Ibraim Aidana

1. Introduction and Objectives

This project integrates a line sensor, an ESP32 microcontroller, a motor driver, a DC motor and ROS 2 into a single sensing-to-actuation chain. The Amperka Octoliner, an eight-channel reflectance sensor that distinguishes a dark line from a light background, serves as the input. The ESP32 reads all eight channels over I²C, streams them to a ROS 2 computer over USB serial, and drives the motor according to the commands ROS 2 sends back.

The objectives were to interface the Octoliner with the ESP32, transmit all eight readings, control a DC motor through a driver, establish bidirectional serial communication with ROS 2, and implement at least two ROS 2 nodes (sensor data and motor commands). The resulting signal flow is:

Octoliner → ESP32 → USB Serial → ROS 2 Nodes → Motor Command → Motor Driver → DC Motor

2. Components

| Component | Role in the system |
| --- | --- |
| ESP32 development board | Main controller: reads the sensor, talks to ROS 2 and switches the motor driver. |
| Amperka Octoliner | Eight-channel analog line sensor connected over I²C. |
| Motor driver (ZK-5AD or equivalent) | Converts low-current ESP32 logic signals into motor drive current. |
| DC motor | Actuator converting electrical energy into rotation. |
| Two Li-ion batteries | Motor power supply, routed through the driver/regulation circuitry. |
| Breadboard, jumper wires, USB cable | Prototype wiring (power, ground, I²C, control) and the ROS 2 serial link. |

3. System Design

Hardware architecture. The Octoliner connects to the ESP32 via I²C and is read with the Octoliner library. Because ESP32 GPIO pins cannot supply the current a DC motor needs, the ESP32 only sends logic-level control signals; the motor driver takes power from the batteries and drives the motor. The pin assignment and control logic are summarised below.

Table 1. Pin assignment

| ESP32 Pin | Motor Driver Pin | Function |
| --- | --- | --- |
| GPIO18 | D0 | Motor forward control |
| GPIO19 | D1 | Motor stop / direction control |

Table 2. Motor control logic

| D0 | D1 | Motor State |
| --- | --- | --- |
| LOW | LOW | Stop |
| HIGH | LOW | Forward |
| LOW | HIGH | Reverse (if supported by the driver) |
| HIGH | HIGH | Not used in this project |

Power and wiring rules. The ESP32, Octoliner and motor driver share a common ground. The motor is powered only through the driver, never from a GPIO pin, and battery voltage is never applied to the ESP32 3.3 V pin; a suitable regulator or driver power input is used instead. Motor power wiring is routed away from sensor wiring where possible to reduce electrical noise.

4. ROS 2 Communication Design

ROS 2 nodes exchange messages through topics. Serial data from the ESP32 is converted into ROS 2 messages, and ROS 2 motor commands are converted back into serial strings.

| Node | Topic | Message type | Function |
| --- | --- | --- | --- |
| 1. Sensor / serial interface | /octoliner_data | std_msgs/msg/Int32MultiArray | Parses S,v0,…,v7 packets and publishes the eight values. |
| 2. Motor command publisher | /motor_command | std_msgs/msg/String | Sends FORWARD or STOP to the ESP32 over serial. |
| 3. Processing (optional) | sub + pub | — | Detects the line from sensor data and publishes FORWARD/STOP automatically. |

A typical sensor packet is S,125,130,142,500,620,580,150,120, where higher values correspond to the channels over the line.

5. ESP32 Program

At start-up the program creates the Octoliner object at I²C address 42, initialises it with a sensitivity of 208, configures GPIO18/19 as outputs and stops the motor to prevent unexpected movement. In the main loop, readMotorCommand() reads one serial line, trims whitespace and calls motorForward() (D0 HIGH, D1 LOW) or motorStop() (both LOW). sendSensorData() then prints the letter S followed by the eight analog readings and a newline, which marks the end of each packet for the ROS 2 node. A 200 ms delay gives about five updates per second, sufficient for a classroom demonstration.

| #include <Octoliner.h> Octoliner octoliner(42); // I2C address 42 #define MOTOR_D0 18 // GPIO18 -> ZK-5AD D0 #define MOTOR_D1 19 // GPIO19 -> ZK-5AD D1 void motorForward() { digitalWrite(MOTOR_D0, HIGH); digitalWrite(MOTOR_D1, LOW); } void motorStop() { digitalWrite(MOTOR_D0, LOW); digitalWrite(MOTOR_D1, LOW); } void setup() { Serial.begin(115200); // USB link to ROS 2 Serial.setTimeout(20); pinMode(MOTOR_D0, OUTPUT); pinMode(MOTOR_D1, OUTPUT); motorStop(); // safe start-up octoliner.begin(); octoliner.setSensitivity(208); } | void readMotorCommand() { if (Serial.available()) { String command = Serial.readStringUntil('\n'); command.trim(); if (command == "FORWARD") motorForward(); else if (command == "STOP") motorStop(); } } void sendSensorData() { // S,v0,...,v7 Serial.print("S"); for (uint8_t i = 0; i < 8; i++) { Serial.print(","); Serial.print(octoliner.analogRead(i)); } Serial.println(); } void loop() { readMotorCommand(); sendSensorData(); delay(200); // ~5 updates/s } |
| --- | --- |

6. Testing Procedure

- Wire the Octoliner to the ESP32 (power, ground, SDA, SCL) and GPIO18/GPIO19 to driver inputs D0/D1.

- Connect the motor to the driver outputs and the batteries to the power input, checking polarity first.

- Connect the ESP32 to the ROS 2 computer by USB, upload the code and check output in the serial monitor.

- Start the ROS 2 sensor node and move black and white surfaces under the Octoliner to observe the readings change.

- Publish FORWARD and confirm the motor rotates; publish STOP and confirm it stops.

ros2 topic echo /octoliner_data ros2 topic pub --once /motor_command std_msgs/msg/String "{data: 'FORWARD'}" ros2 topic pub --once /motor_command std_msgs/msg/String "{data: 'STOP'}"

7. Results and Discussion

The system demonstrates a complete path from sensor input to actuator output. The Octoliner continuously measures reflected infrared light at eight positions; the ESP32 formats these readings as serial packets that ROS 2 can display, record or process. When ROS 2 publishes FORWARD, the ESP32 drives GPIO18 HIGH and the motor runs; on STOP, both control pins go LOW and the motor halts. The project therefore meets the assignment requirements: a DC motor as actuator, the Octoliner as sensor, and two ROS 2 nodes handling sensor data and motor commands.

8. Conclusion

This project built a ROS 2-based sensor and actuator system from an ESP32, an Amperka Octoliner, a motor driver, a DC motor and two Li-ion batteries. Eight-channel readings travel from the sensor to the ESP32 over I²C and on to ROS 2 over USB serial, while motor commands flow back the same link. The implementation covers the full robotic control chain of sensing, communication, ROS 2 processing, command transmission and actuation.